# ICSI404 – Assignment 4 – Assembler – ICSI311 Redux!

## Introduction

As you can imagine, hand-coding strings of bits is very tedious. It can be done, but it is not a lot of fun. Professionals nearly never do it. They instead write an assembler.

Fortunately, writing an assembler is very easy – far easier than the interpreter that we wrote in ICSI311, but the steps are the same!

First, let’s talk about what assembly looks like. Typically, the opcode is first. The addressing mode is inferred from the arguments. Consider our SIA Spring 2026 document.

Should we prefer compound words, or should we split the words? shiftRight vs. right shift vs. shift right. As you can probably imagine, this is actually pretty easy to handle in the parser – when you see certain words, require a second word type (shift requires left or right). Let’s go the simpler route and keep all of the instructions as single words. After the keyword, there are 0, 1 or 2 values. A value is either a register “r0” – “r31” or a number (including negatives, but no decimals).

Unlike most programming languages, the format of assembly is so easy that we don’t need a “real” lexer. We can use string splitting! You will want to use hash maps like we did in ICSI311, but here, we map opcodes to bit strings. For example, “halt”  “0000”. You can do something similar with registers.

## Details

We have three different instruction formats to deal with. Most instructions are EITHER 2R or immediate. It’s easy to detect the different – is the second split a register or not.

For Call/Return format, you expect only one parameter, and it will always be a number.

finalOutput() – the assembler outputs an array of strings, each 16 bits (16 0’s or 1’s). This isn’t the format that memory.load() requires. finalOutput() merges lines 0 & 1, 1&2, etc. If there are an odd number of lines, it adds a halt (0000000000000000) to the end. The result is 32 bit lines.

For this assignment, you don’t need to use the Bit and Word classes. You certainly can.

Examples:

add r1 r2

From this instruction we can tell that we should be using 2R format.

| 5 bits | 1 bit | 5 bits | 5 bits |
| --- | --- | --- | --- |
| OPCODE | Format (0) | Source register | Destination register |

Add is opcode 1, so that translates to 00001

Format bit is 0.

Source is register 1, so 00001; destination is register 2, so 00010

This instruction assembles to 0000100000100010

load 15 R7

From this instruction we can tell that we should be using the immediate format.

| 5 bits | 1 bit | 5 bits | 5 bits |
| --- | --- | --- | --- |
| OPCODE | Format (1) | Signed immediate value | Destination register |

Load is opcode 18, so that translates to 10010

Format bit is 1.

Immediate value is 01111, the register is 00111

This instruction assembles to 1001010111100111

Rubric

| Assemble | 25 |
| --- | --- |
| testInstructions | 75 |

Checklist Before Submission

I am submitting only .java files. Other files will be ignored.

I am submitting all of the files that are necessary to build and grade.

My project builds

I did not copy another student’s work (or my own from a previous semester)

I did not use AI to generate this code.

If applicable, all of my bugs from previous assignments are fixed.