# ICSI404 – Assignment 5 – Processor

## Introduction

With memory and the ALU built, we are actually a long way toward the complete computer! What does the processor do? It uses the PC (program counter) to keep track of which instruction it is working on. It gets that instruction from memory, “does it”, then moves to the next instruction.

Writing this code is a little different then writing most programs. We are simulating hardware, so there are not a lot of loops. You should be using no Java math (except – I have some for loops with a ++ index). You will have several big switch statements – decode, execute and store (see below) will have different behavior based on opcode. It is OK to extract the opcode in bits and use an integer for your switch statement. Make sure that you are using assign() and copy() on your word32s. Java assign (=) can cause you subtle bugs.

## Details

Run() is the only public method of the processor. It runs in a loop, calling fetch(), decode(), execute() and store() until “halt” is run.

### Fetch

Fetch reads an instruction from memory. Ours is a little tricky, because a single read() gives us 2 instructions. Make sure that you handle this efficiently (not reading every fetch() call). You will need to keep track of where you are in that process. Fetch() was the reason that we write getTopHalf() and getBottomHalf() on Word32.

### Decode

Decode’s “job” is to get the information ready for execution. You will need two members: op1 and op2 that will hold the data for each instruction. For instructions that use registers, copy() from the register into op1/op2. For immediate values, copy and sign extend the immediate value. Note that some instructions have different formats, so you have to be careful.

### Execute

Execute “does the work”. Depending on the opcode, that could be invoking the ALU, going out to memory (load/store), or other work.

### Store

For most instructions, store() is simply copying the result value into the appropriate register. The branches, call and return change the PC. Store() is also the correct place to move the PC ahead when necessary, so that the next fetch() gets the right address.

Other things – you will need a stack for call/return. You can use Java’s built-in stack for this. Of course, real CPUs don’t do this, but it is a small issue. You will need to create 32 registers (an array makes the most sense). You will have several private members as well.  This is a little bit of a big assignment – my solution is about 250 lines. Note that the tests are complete assembly language programs!

Rubric

| testAdd | 10 |
| --- | --- |
| testPower | 40 |
| testFibonacci | 50 |

Checklist Before Submission

I am submitting only .java files. Other files will be ignored.

I am submitting all of the files that are necessary to build and grade.

My project builds

I did not copy another student’s work (or my own from a previous semester)

I did not use AI to generate this code.

If applicable, all of my bugs from previous assignments are fixed.